## Qnect Connection Activity

## Video 1 Comprehension Questions

- Briefly explain the capabilities of Qnect as outlined in the video you just watched.

- What parties within a building design team (i.e. structural designer, architect, owner, fabricator, contractor, etc.) would most likely use Qnect?

- What type of structural information is provided in the Downers Grove Tekla Structures model? List at least 3 pieces of information that can be obtained from the Tekla model to be used with Qnect.

## Qnect Connection Activity

## Video 2 Comprehension Questions

- How is a bay defined? What is the bay size for this building in the East-West direction?

- Why is it desirable to have uniform and orthogonal bay sizes?

- What is the purpose of the embed plate at the end of the floors at the 1st floor level?

- Explain the difference between a gravity and lateral system.

- Members in this Tekla model are color coded as grey, light purple, dark purple, green, and yellow. Name the member or system that the following colors represent, and describe their role in the load path of this building.

- Why is it that you typically see steel truss joists to support a roof, but less commonly to support a floor level?

- What observations are made about the location of the Lateral Force Resisting Frames (LFRS) in this building?

- What role does the composite floor (concrete slab on metal deck) have when we consider the load path of wind load making its way down to the foundation of the building?

## Qnect Connection Activity

## Video 3 Comprehension Questions

- Briefly summarize the typical three scenarios regarding which responsible parties would perform the connection design.

What is the Engineer of Record’s (EOR) responsibility for each of these cases?

- If the connection design is delegated to a third party, give four examples of the information that must be provided by the EOR in order for the work to be performed.

- What type of information is included in typical connection details? Provide at least two examples.

- How are connection demands typically indicated in the construction drawings?

## Qnect Connection Activity

## Video 4 Comprehension Questions

- Describe why it is desirable to install two erection bolts as a preliminary step during the construction of a steel building.

- What is “plumbing” of the structure?

- Explain the difference between a field-welded and shop-welded connection. Which is generally preferred?

- Describe the constructability challenges if specifying a shop-welded/field bolted beam-to-girder double angle connection.

- From the list of connections provided, indicate whether the connection would typically be considered a connection within the gravity frame (GF) or lateral frame (LF). Indicate GF or LF below.

Shear tab connection __________

Double angle connection ____________

Moment connection ____________

Unstiffened seated connection _______

Beam to brace connection ___________

Single angle connection __________

Brace to column connection _________

- For the connection shown, circle the cope. Briefly explain the purpose of coping

## Qnect Connection Activity

## Beam–to–Girder Connections (to be completed after Video 5)

Refer to the Downers Grove Qnect model and find the beam to spandrel connection at level 2 that is closest to Column A-1 for this study.

- Start with a shear tab connection with a 3/8” plate and ¾” diameter A325N bolts, with 3” spacing, eccentricity of 3”, and 1.5” edge distances. Obtain the following information from Qnect and also complete the shaded region of the table.

Beam Size: _____________

Girder Size: ________________

Factored Connection Demand: ______________

Values obtained from Qnect

| Connection Type: | Shear Tab | Single Angle – bolted/welded | Double Angle – bolted/bolted | Double Angle – welded/bolted |
| --- | --- | --- | --- | --- |
| # of bolts req’d |  |  |  |  |
| Weld size, as applicable |  |  |  |  |
| Plate or angle thickness |  |  |  |  |
| Capacity of the connection |  |  |  |  |
| Controlling limit state |  |  |  |  |
| Connection cost |  |  |  |  |

- Using Qnect, re-design the connection for the other three connection types in the table and input that information.

- For the bolted-bolted double angle connection, review the Qnect connection calculations and validate the capacity determined from Qnect by using AISC design tables and limit state calculations. If the connection design table is not a match with the Qnect results, explain why. Remember that the design table also does not consider member limit states (i.e. coped beam checks, bearing on the beam web, etc.) so you may need to calculate these member limit states independently of simply using the design aid.

- Select the connection type with the lowest capacity. Modify one of the pertinent preference settings in Qnect in order to increase that connection capacity by 20%. i.e. – if bolt shear is the controlling limit state, modify the connection to use a higher grade bolt or larger diameter bolt (see General Bolt Preferences in your job on the Qnect Dashboard). Explain what modifications you made and how this influenced the overall connection capacity.

Qnect Connection Activity

Beam–to–Column Connections (to be completed after Video 6)

- Explain three of the issues that may pertain to beam-to-column web connections that are not prevalent to beam-to-column flange connections.

- Show screenshots of two different connection types designed using Qnect that achieve adequate capacity for the beam-to-column web connection at level 2, column C-2. Compare the connection capacities determined from Qnect. Then discuss at least one pro and con of each configuration.